Problem 15.30

The following model, based on a simplification of the Arrhenius equation, is frequently used in environmental engineering to parameterize the effect of temperature, T (°C), on pollutant decay rates, k (per day),

k = k20 θ T−20

where the parameters k20 = the decay rate at 20 °C, and θ = the dimensionless temperature dependence coefficient. The following data are collected in the laboratory

| T (°C) | 6 | 12 | 18 | 24 | 30 |
| --- | --- | --- | --- | --- | --- |
| k (per d) | 0.15 | 0.20 | 0.32 | 0.45 | 0.70 |

(a) Use a transformation to linearize this equation and then employ linear regression to estimate k20 and θ.

(b) Employ nonlinear regression to estimate the same parameters.

For both (a) and (b) employ the equation to predict the reaction rate at T = 17 °C.