Solution 15.14

(a) The model can be linearized by inverting it,

If this model is valid, a plot of 1/v0 versus 1/[S]3 should yield a straight line with a slope of K/km and an intercept of 1/km. The slope and intercept can be implemented in MATLAB using the M-file function linregr (Fig. 14.15),

>> S = [.01 .05 .1 .5 1 5 10 50 100];

>> v0 = [6.078e-11 7.595e-9 6.063e-8 5.788e-6 1.737e-5 2.423e-5 ...

2.43e-5 2.431e-5 2.431e-5];

>> a = linregr(1./S.^3,1./v0)

a =

16453        41400

These results can then be used to compute km and K,

>> km=1/a(2)

km =

2.4155e-005

>> K=km*a(1)

K =

0.39741

Thus, the best-fit model is

Solution continued on next page...

The fit along with the data can be displayed graphically. We will use a log-log plot because of the wide variation of the magnitudes of the values being displayed,

>> v0p = km*S.^3./(K+S.^3);

>> loglog(S,v0,'o',S,v0p)

(b) An M-file function must be created to compute the sum of the squares,

function f = fSSR (a,Sm,v0m)

v0p = a(1)*Sm.^3./(a(2)+Sm.^3);

f = sum((v0m-v0p).^2);

The data can then be entered as

>> S = [.01 .05 .1 .5 1 5 10 50 100];

>> v0 = [6.078e-11 7.595e-9 6.063e-8 5.788e-6 1.737e-5 ...

2.423e-5 2.43e-5 2.431e-5 2.431e-5];

The minimization of the function is then implemented by

>> a = fminsearch(@fSSR, [2e-5, 1], [], S, v0)

a =

2.431e-005      0.39976

The best-fit model is therefore

Solution continued on next page...

The fit along with the data can be displayed graphically. We will use a log-log plot because of the wide variation of the magnitudes of the values being displayed,

>> v0p = a(1)*S.^3./(a(2)+S.^3);

>> loglog(S,v0,'o',S,v0p)