Solution 15.24

Nonlinear regression can be used to evaluate the coefficients. First, a function can be set up to compute the sum of the squares of the residuals,

function f = fSSR1524(a,Nm,Stressm)

Stressp = a(1)*Nm.^a(2);

f = sum((Stressm-Stressp).^2);

Then, the fminsearch function can be employed to determine the coefficients,

clear, clc,clf,format short g

N=[1 10 100 1000 10000 100000 1000000]; Stress=[1100 1000 925 800 625 550 420];

a = fminsearch(@fSSR1524, [10,2], [], N, Stress)

Np=logspace(0,6);

Stressp = a(1)*Np.^a(2);

subplot(1,2,1)

plot(N,Stress,'o',Np,Stressp)

xlabel('N'),ylabel('Stress')

subplot(1,2,2)

semilogx(N,Stress,'o',Np,Stressp)

xlabel('log(N)'),ylabel('Stress')

a =

1150.3    -0.063082