Solution 15.25

Nonlinear regression can be used to evaluate the coefficients. First, a function can be set up to compute the sum of the squares of the residuals,

function f = fSSR1525(a,Tm,Viscm)

Viscp = a(1)*Tm.^a(2);

f = sum((Viscm-Viscp).^2);

Then, the fminsearch function can be employed to determine the coefficients,

clear,clc,clf,format short g

T=[26.67 93.33 148.89 315.56]; Visc=[1.35 0.085 0.012 0.00075];

a = fminsearch(@fSSR1525, [10,2], [], T, Visc)

Tp=[25:320];

Viscp = a(1)*Tp.^a(2);

plot(T,Visc,'o',Tp,Viscp)

xlabel('T'),ylabel('Viscosity')

a =

2416.2       -2.281