Problem 16.11

Duplicate Example 16.3, but for 64 points sampled at a rate of Δt = 0.01 s from the function

f (t) = cos[2π (12.5)t] + cos[2π (25)t]

Use fft to generate a DFT of these values and plot the results.

Problem Statement of Example 16.3:

Apply the MATLAB fft function to determine the discrete Fourier transform for a simple sinusoid:

f (t) = 5 + cos(2π(12.5)t ) + sin(2π(18.75)t )

Generate 8 equispaced points with Δt = 0.02 s. Plot the result versus frequency.