Solution 16.3

The angular frequency can be computed as 0 = 2/24 = 0.261799. The various summations required for the normal equations can be set up as

| t | y | cos(0t) | sin(0t) | sin(0t)cos(0t) | cos2(0t) | sin2(0t) | ycos(0t) | ysin(0t) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 7.6 | 1.00000 | 0.00000 | 0.00000 | 1.00000 | 0.00000 | 7.60000 | 0.00000 |
| 2 | 7.2 | 0.86603 | 0.50000 | 0.43301 | 0.75000 | 0.25000 | 6.23538 | 3.60000 |
| 4 | 7 | 0.50000 | 0.86603 | 0.43301 | 0.25000 | 0.75000 | 3.50000 | 6.06218 |
| 5 | 6.5 | 0.25882 | 0.96593 | 0.25000 | 0.06699 | 0.93301 | 1.68232 | 6.27852 |
| 7 | 7.5 | –0.25882 | 0.96593 | –0.25000 | 0.06699 | 0.93301 | –1.94114 | 7.24444 |
| 9 | 7.2 | –0.70711 | 0.70711 | –0.50000 | 0.50000 | 0.50000 | –5.09117 | 5.09117 |
| 12 | 8.9 | –1.00000 | 0.00000 | 0.00000 | 1.00000 | 0.00000 | –8.90000 | 0.00000 |
| 15 | 9.1 | –0.70711 | –0.70711 | 0.50000 | 0.50000 | 0.50000 | –6.43467 | –6.43467 |
| 20 | 8.9 | 0.50000 | –0.86603 | –0.43301 | 0.25000 | 0.75000 | 4.45000 | –7.70763 |
| 22 | 7.9 | 0.86603 | –0.50000 | –0.43301 | 0.75000 | 0.25000 | 6.84160 | –3.95000 |
| 24 | 7 | 1.00000 | 0.00000 | 0.00000 | 1.00000 | 0.00000 | 7.00000 | 0.00000 |
| sum | 84.8 | 2.31784 | 1.93185 | 0.00000 | 6.13397 | 4.86603 | 14.94232 | 10.18401 |

The normal equations can be assembled as

This system can be solved for A0 = 8.0270, A1 = –0.59717, and B1 = –1.09392. Therefore, the best-fit sinusoid is

The result can also be expressed in the alternate form of Eq. 16.2 by computing the amplitude and the phase shift,

Solution continued on next page...

Therefore, the fit can also be expressed as

Consequently, the mean is 8.0270 and the amplitude is 1.2463. To determine the time of the maximum, inspection of Fig. 16.3 indicates that a positive phase shift represents the time prior to midnight that the peak occurs. Therefore the time of the maximum can be computed as

tmax = 24 – 7.9087 = 16.0913 hrs

This is equal to about 16:05:29 or 4:05:29 PM. The data and the model can be plotted as

The same calculation can be implemented with MATLAB. The coefficients can also be determined with the following script:

clear,clc,clf,format compact

w0 = 2*pi/24;

t = [0 2 4 5 7 9 12 15 20 22 24]';

pH = [7.6 7.2 7 6.5 7.5 7.2 8.9 9.1 8.9 7.9 7]';

Z = [ones(size(t)) cos(w0*t) sin(w0*t)];

a = (Z'*Z)\(Z'*pH)

Sr = sum((pH - Z*a).^2);

syx = sqrt(Sr/(length(t)-length(a)))

tp = [0:24]; pHp = a(1)+a(2)*cos(w0*tp)+a(3)*sin(w0*tp);

plot(t,pH,'o',tp,pHp)

title('Time vs. pH'),xlabel('Time (hours)'),ylabel('pH')

mean=a(1),theta = atan2(-a(3),a(2))*24/(2*pi)

Amplitude = sqrt(a(2)^2+a(3)^2)

time_Max_pH = 24-theta

Solution continued on next page...

Output:

a =

8.0270

-0.5972

-1.0939

syx =

0.3447

mean =

8.0270

theta =

7.9087

Amplitude =

1.2463

time_Max_pH =

16.0913