Problem 17.15

The specific volume of a superheated steam is listed in steam tables for various temperatures.

| T, °C | 370 | 382 | 394 | 406 | 418 |
| --- | --- | --- | --- | --- | --- |
| υ, L/kg | 5.9313 | 7.5838 | 8.8428 | 9.796 | 10.5311 |

Determine υ at T = 400 ºC.