Problem 17.16

The vertical stress σz under the corner of a rectangular area subjected to a uniform load of intensity q is given by the solution of Boussinesq’s equation:

Because this equation is inconvenient to solve manually, it has been reformulated as

σz = q fz(m, n)

where fz(m, n) is called the influence value, and m and n are dimensionless ratios, with m = a∕z and n = b∕z and a and b are defined in Fig. P17.16. The influence value is then tabulated, a portion of which is given in Table P17.16. If a = 4.6 and b = 14, use a third-order interpolating polynomial to compute σz at a depth 10 m below the corner of a rectangular footing that is subject to a total load of 100 t (metric tons). Express your answer in tonnes per square meter. Note that q is equal to the load per area.

Table P17.16

| m | n =1.2 | n=1.4 | n=1.6 |
| --- | --- | --- | --- |
| 0.1 | 0.02926 | 0.03007 | 0.03058 |
| 0.2 | 0.05733 | 0.05894 | 0.05994 |
| 0.3 | 0.08323 | 0.08561 | 0.08709 |
| 0.4 | 0.10631 | 0.10941 | 0.11135 |
| 0.5 | 0.12626 | 0.13003 | 0.13241 |
| 0.6 | 0.14309 | 0.14749 | 0.15027 |
| 0.7 | 0.15703 | 0.16199 | 0.16515 |
| 0.8 | 0.16843 | 0.17389 | 0.17739 |