Solution 17.8

MATLAB can be used to generate a cubic polynomial through the first 4 points in the table.

x = [1 2 3 4];

fx = [3.6 1.8 1.2 0.9];

p = polyfit(x,fx,3)

p =

-0.1500    1.5000   -5.2500    7.5000

Therefore, the roots problem to be solved is

or

The fzero function can then be used to determine the root

p(1,4)=p(1,4)-1.7

fx=@(x,p) polyval(p,x);

x = fzero(fx,mean(x),[],p)

The result is

p =

-0.15          1.5        -5.25          5.8000

x =

2.1009

Thus, interpolated value of x at y = 1.7 is 2.1009.