Problem 18.13

The drag coefficient for spheres such as sporting balls is known to vary as a function of the Reynolds number Re, a dimensionless number that gives a measure of the ratio of inertial forces to viscous forces:

where ρ = the fluid’s density (kg/m3), V = its velocity (m/s), D = diameter (m), and μ = dynamic viscosity (N · s/m2). Although the relationship of drag to the Reynolds number is sometimes available in equation form, it is frequently tabulated. For example, the following table provides values for a smooth spherical ball:

| Re(×10−4) | 2 | 5.8 | 16.8 | 27.2 | 29.9 | 33.9 | 36.3 | 40 | 46 | 60 | 100 | 200 | 400 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CD | 0.52 | 0.52 | 0.52 | 0.5 | 0.49 | 0.44 | 0.18 | 0.074 | 0.067 | 0.08 | 0.12 | 0.16 | 0.19 |

(a) Develop a MATLAB function that employs an appropriate interpolation function to return a value of CD as a function of the Reynolds number. The first line of the function should be

function CDout = Drag(ReCD,ReIn)

where ReCD = a 2-row matrix containing the table, ReIn = the Reynolds number at which you want to estimate the drag, and CDout = the corresponding drag coefficient.

(b) Write a script that uses the function developed in part (a) to generate a labeled plot of the drag force versus velocity (recall Sec. 1.4). Use the following parameter values for the script: D = 22 cm, ρ = 1.3 kg/m3, and μ = 1.78 × 10−5 Pa · s. Employ a range of velocities from 4 to 40 m/s for your plot.