Problem 18.15

The U.S. Standard Atmosphere specifies atmospheric properties as a function of altitude above sea level. The following table shows selected values of temperature, pressure, and density

| Altitude (km) | T (°C) | p (atm) | ρ (kg/m3) |
| --- | --- | --- | --- |
| −0.5 | 18.4 | 1.0607 | 1.2850 |
| 2.5 | −1.1 | 0.73702 | 0.95697 |
| 6 | −23.8 | 0.46589 | 0.66015 |
| 11 | −56.2 | 0.22394 | 0.36481 |
| 20 | −56.3 | 0.054557 | 0.088911 |
| 28 | −48.5 | 0.015946 | 0.025076 |
| 50 | −2.3 | 7.8721 × 10−4 | 1.0269 × 10−3 |
| 60 | −17.2 | 2.2165 × 10−4 | 3.0588 × 10−4 |
| 80 | −92.3 | 1.0227 × 10−5 | 1.9992 × 10−5 |
| 90 | −92.3 | 1.6216 × 10−6 | 3.1703 × 10−6 |

Develop a MATLAB function, StdAtm, to determine values of the three properties for a given altitude. Base the function on the pchip option for interp1. If the user requests a value outside the range of altitudes, have the function display an error message and terminate the application. Use the following script as the starting point to create a 3-panel plot of altitude versus the properties as depicted in Fig. P18.15.

Problem continued on next page...

% Script to generate a plot of temperature, pressure and density

% for the U.S. Standard Atmosphere

clc, clf

z = [−0.5 2.5 6 11 20 28 50 60 80 90];

T = [18.4 −1.1 −23.8 −56.2 −56.3 −48.5 −2.3 −17.2 −92.3 −92.3];

p = [1.0607 0.73702 0.46589 0.22394 0.054557 0.015946 ...

7.8721e − 4 2.2165e − 4 1.02275e − 05 1.6216e − 06];

rho = [1.285025 0.95697 0.6601525 0.364805 0.0889105 ...

0.02507575 0.001026918 0.000305883 0.000019992

3.1703e − 06];

zint = [− 0.5:0.1:90];

for i = 1:length(zint)

[Tint(i),pint(i),rint(i)] = StdAtm(z,T,p,rho,zint(i));

end

% Create plot

Te = StdAtm(z,T,p,rho,−1000);