Problem 18.16

Felix Baumgartner ascended to 39 km in a stratospheric balloon and made a free-fall jump rushing toward earth at supersonic speeds before parachuting to the ground. As he fell, his drag coefficient changed primarily because the air density changed. Recall from Chap. 1 that the terminal velocity, υterminal (m/s), of a free-falling object can be computed as

where g = gravitational acceleration (m/s2), m = mass (kg), and cd = drag coefficient (kg/m). The drag coefficient can be computed as

cd = 0.5ρACd

where ρ = the fluid density (kg/m3), A = the projected area (m2), and Cd = a dimensionless drag coefficient. Note that the gravitational acceleration, g (m/s2), can be related to elevation by

g = 9.806412 − 0.003039734z

where z = elevation above the earth’s surface (km) and the density of air, ρ (kg/m3), at various elevations can be tabulated as

| z (km) | ρ (kg/m3) | z (km) | ρ (kg/m3) | z (km) | ρ (kg/m3) |
| --- | --- | --- | --- | --- | --- |
| −1 | 1.347 | 6 | 0.6601 | 25 | 0.04008 |
| 0 | 1.225 | 7 | 0.5900 | 30 | 0.01841 |
| 1 | 1.112 | 8 | 0.5258 | 40 | 0.003996 |
| 2 | 1.007 | 9 | 0.4671 | 50 | 0.001027 |
| 3 | 0.9093 | 10 | 0.4135 | 60 | 0.0003097 |
| 4 | 0.8194 | 15 | 0.1948 | 70 | 8.283 × 10−5 |
| 5 | 0.7364 | 20 | 0.08891 | 80 | 1.846 × 10−5 |

Assume that m = 80 kg, A = 0.55 m2, and Cd = 1.1. Develop a MATLAB script to create a labeled plot of terminal velocity versus elevation for z = [0:0.5:40]. Use a spline to generate the required densities needed to construct the plot.