Solution 18.10

(a) Here is a MATLAB script to develop the spline and plot it along with the data:

x=[0 2 4 7 10 12]; y=[20 20 12 7 6 6];

xx=linspace(0,12);

yy=spline(x,y,xx);

plot(x,y,'o',xx,yy)

Here is the command to make the prediction:

>> yp=spline(x,y,1.5)

yp =

21.3344

Solution continued on next page...

(b) To prescribe zero first derivatives at the end knots, the y vector is modified so that the first and last elements are set to the desired values of zero. The plot and the prediction both indicate that there is less overshoot between the first two points because of the prescribed zero slopes.

>> yd=[0 y 0];

>> yy=spline(x,yd,xx);

>> plot(x,y,'o',xx,yy)

>> yy=spline(x,yd,1.5)

yy =

20.5701