Problem 19.23

A spherical tank has a circular orifice in its bottom through which the liquid flows out (Fig. P19.23). The following data is collected for the flow rate through the orifice as a function of time:

| t, s | 0 | 500 | 1000 | 1500 | 2200 | 2900 | 3600 | 4300 | 5200 | 6500 | 7000 | 7500 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Q, m3/hr | 10.55 | 9.576 | 9.072 | 8.640 | 8.100 | 7.560 | 7.020 | 6.480 | 5.688 | 4.752 | 3.348 | 1.404 |

Write a script with supporting functions

(a) to estimate the volume of fluid (in liters) drained over the entire measurement period and

(b) to estimate the liquid level in the tank at t = 0 s. Note that r = 1.5 m.