Solution 19.23

The volume flowing out of the tank can be computed by integration with conversions to express the result in liters.

The volume of a partially filled spherical tank

Thus, if we know V and r, determining H can be expressed as a roots problem

Here is the script that solves the problem

clear,clc

format short g, format compact

t=[0  500 1000  1500  2200  2900  3600  4300  5200  6500  7000  7500];

Q=[10.55  9.576 9.072 8.640 8.100 7.560 7.020 6.480 5.688 4.752 3.348 1.404];

V_liters=1e3/3600*trapz(t,Q)

r=1.5;

V_m3=V_liters/1e3

Vfull=4/3*pi*r^3

f=@(H) pi/3*H^2*(3*r-H)-V_m3;

Hinitial=fzero(f,2*r)

V_liters =

14101

V_m3 =

14.101

Vfull =

14.137

Hinitial =

2.9117