Solution 20.12

(a)

clear,clc,format compact

F1=@(t) (sin(2*pi*t)).^2;

Irms=sqrt(integral(F1,0,1))

R=5; P=Irms^2*R

Irms =

0.7071

P =

2.5000

(b)

ib=@(t) sin(2*pi*t);

Vb=@(t) 5*ib(t)-1.25*ib(t).^2;

Pb=@(t) ib(t).*Vb(t);

P=integral(Pb,0,1)

P =

2.5000

Interestingly, the power is identical. The reason for this can be seen by inspecting each of the power functions. For (a), the power function is

For (b), it is

Solution continued on next page...

A plot can be developed of both functions along with their difference.

t=linspace(0,1);

Pa=@(t) 5*(sin(2*pi*t)).^2;

P1=Pa(t); P2=Pb(t);

delta=P2-P1;

subplot(2,1,1),plot(t,P1,t,P2,'--')

legend('(a)','(b)'),title('Power versus time')

subplot(2,1,2),plot(t,delta)

title('difference')

As can be seen, the difference is symmetrical across the period. Therefore, the positive and negative discrepancies cancel.