Problem 21.10

Develop an M-file function that computes first and second derivative estimates of order O(h2) based on the formulas in Fig. 21.3 through Fig. 21.5. The function’s first line should be set up as

function [dydx, d2ydx2] = diffeq(x,y)

where x and y are input vectors of length n containing the values of the independent and dependent variables, respectively, and dydx and dy2dx2 are output vectors of length n containing the first- and second-derivative estimates at each value of the independent variable. The function should generate a plot of dydx and dy2dx2 versus x.

Have your M-file return an error message if

(a) the input vectors are not the same length or

(b) the values for the independent variable are not equally spaced.

Test your program with the data from Prob. 21.11.

Data from Problem 21.11:

| t, s | 0 | 25 | 50 | 75 | 100 | 125 |
| --- | --- | --- | --- | --- | --- | --- |
| y, km | 0 | 32 | 58 | 78 | 92 | 100 |

Problem continued on next page...

Problem continued on next page...