Problem 21.22

Fick’s first diffusion law states that

where mass flux = the quantity of mass that passes across a unit area per unit time (g/cm2/s), D = a diffusion coefficient (cm2/s), c = concentration (g/cm3), and x = distance (cm). An environmental engineer measures the following concentration of a pollutant in the pore waters of sediments underlying a lake (x = 0 at the sediment-water interface and increases downward):

| x, cm | 0 | 1 | 3 |
| --- | --- | --- | --- |
| c, 10−6 g/cm3 | 0.06 | 0.32 | 0.6 |

Use the best numerical differentiation technique available to estimate the derivative at x = 0. Employ this estimate in conjunction with Eq. (P21.22) to compute the mass flux of pollutant out of the sediments and into the overlying waters (D = 1.52 × 10−6 cm2/s). For a lake with 3.6 × 106 m2 of sediments, how much pollutant would be transported into the lake over a year’s time?