Problem 21.24

Fourier’s law is used routinely by architectural engineers to determine heat flow through walls. The following temperatures are measured from the surface (x = 0) into a stone wall:

| x, m | 0 | 0.08 | 0.16 |
| --- | --- | --- | --- |
| T, °C | 20.2 | 17 | 15 |

If the flux at x = 0 is 60 W/m2, compute k.