Problem 21.33

The specific heat at constant pressure cp [J/(kg · K)] of an ideal gas is related to enthalpy by

where h = enthalpy (kJ/kg) and T = absolute temperature (K). The following enthalpies are provided for carbon dioxide (CO2) at several temperatures. Use these values to determine the specific heat in J/(kg · K) for each of the tabulated temperatures. Note that the atomic weights of carbon and oxygen are 12.011 and 15.9994 g/mol, respectively

| T, K | 750 | 800 | 900 | 1000 |
| --- | --- | --- | --- | --- |
| h, kJ/kmol | 29,629 | 32,179 | 37,405 | 42,769 |