Problem 21.35

The sediment oxygen demand [SOD in units of g/(m2 · d)] is an important parameter in determining the dissolved oxygen content of a natural water. It is measured by placing a sediment core in a cylindrical container (Fig. P21.35). After carefully introducing a layer of distilled, oxygenated water above the sediments, the container is covered to prevent gas transfer. A stirrer is used to mix the water gently, and an oxygen probe tracks how the water’s oxygen concentration decreases over time. The SOD can then be computed as

where H = the depth of water (m), o = oxygen concentration (g/m3), and t = time (d).

Based on the following data and H = 0.1 m, use numerical differentiation to generate plots of

(a) SOD versus time and

(b) SOD versus oxygen concentration:

| t, d | 0 | 0.125 | 0.25 | 0.375 | 0.5 | 0.625 | 0.75 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| o, mg/L | 10 | 7.11 | 4.59 | 2.57 | 1.15 | 0.33 | 0.03 |