Solution 21.26

For the equispaced data, we can use the second-order finite divided difference formulas from Figs. 21.3 through 21.5. For example, for the first point, we can use

For the intermediate equispaced points, we can use centered differences. For example, for the second point

For the last point, we can use a backward difference

One of the points (t = 0.3) has unequally spaced neighbors. For this point, we can use Eq. (21.21) to compute the derivative as

x0 = 0.2	      f(x0) = 0.32

x1 = 0.3	      f(x1) = 0.56

x2 = 0.5	      f(x2) = 0.84

We can then multiply these derivative estimates by the inductance. All the results are summarized below:

| t | i | di/dt | VL |
| --- | --- | --- | --- |
| 0 | 0 | 1.6 | 6.4 |
| 0.1 | 0.16 | 1.6 | 6.4 |
| 0.2 | 0.32 | 2 | 8 |
| 0.3 | 0.56 | 2.066667 | 8.266667 |
| 0.5 | 0.84 | 3.6 | 14.4 |
| 0.7 | 2 | 8 | 32 |