Solution 21.31

(a) The pressure drop can be determined by integrating the pressure gradient

After converting the units to meters, a table can be set up holding the data and the integrand. The trapezoidal and Simpson’s rules can then be used to integrate this data as shown in the last column of the table.

| x, m | r, m | integrand | integral | method |
| --- | --- | --- | --- | --- |
| 0 | 0.002 | –7957.75 |  |  |
| 0.02 | 0.00135 | –38333.2 |  |  |
| 0.04 | 0.00134 | –39490.3 | –1338.5392 | Simp 1/3 |
| 0.05 | 0.0016 | –19428.1 |  |  |
| 0.06 | 0.00158 | –20430.6 |  |  |
| 0.07 | 0.00142 | –31315.3 | –713.9319 | Simp 3/8 |
| 0.1 | 0.002 | –7957.75 | –589.0959 | Trap |
|  |  | Sum | –2641.5670 |  |

Therefore, the pressure drop is computed as –2,641.567 N/m2.

(b) The average radius can also be computed by integration as

The numerical evaluations can again be determined by a combination of the trapezoidal and Simpson’s rules.

| x, m | r, m | integral | method |
| --- | --- | --- | --- |
| 0 | 0.00200 |  |  |
| 0.02 | 0.00135 |  |  |
| 0.04 | 0.00134 | 5.83E–05 | Simp 1/3 |
| 0.05 | 0.00160 |  |  |
| 0.06 | 0.00158 |  |  |
| 0.07 | 0.00142 | 4.61E–05 | Simp 3/8 |
| 0.1 | 0.00200 | 5.13E–05 | Trap |
|  | Sum | 0.0001557 |  |

Solution continued on next page...

Therefore, the average radius is 0.0001557/0.1 = 0.001557 m. This value can be used to compute a pressure drop of

Thus, there is less pressure drop if the radius is at the constant mean value.

(c) The average Reynolds number can be computed by first determining the average velocity as

Then the Reynolds number can be computed as