Solution 21.34

The following script determines dc/dt using an M-file that implements Eq. (21.21). Linear regression is then used to fit the log transformed equation. The best-fit parameters are then used to compute the model coefficients n and k with the following script:

t=[0 5 15 30 45];

c=[0.750 0.594 0.420 0.291 0.223];

dcdt=diffuneq(t,c)

p=polyfit(log10(c),log10(-dcdt),1)

n=p(1)

k=10^p(2)

dcdt =

-0.0358   -0.0266   -0.0139   -0.0066   -0.0025

p =

2.1419   -1.1121

n =

2.1419

k =

0.0773