Solution 21.36

(a) The deflection is computed by the following integration,

Here is the numerical evaluation of this integral with the trapezoidal rule.

In addition, an exact, closed form solution can be developed by integrating Eq. P21.36 to give

The following table implements this equation. The first two columns hold the x and  values (Eq. P21.36) and the third column holds the trapezoidal rule evaluation for each interval. The fourth column then holds the summation that represents the deflection,

Solution continued on next page...

| x | (x) | Trap rule | y(x) | ytrue |
| --- | --- | --- | --- | --- |
| 0 | –9.375E–04 | 0.000E+00 | 0.000E+00 | 0.000E+00 |
| 0.125 | –9.277E–04 | –1.166E–04 | –1.166E–04 | –1.168E–04 |
| 0.25 | –8.987E–04 | –1.142E–04 | –2.307E–04 | –2.311E–04 |
| 0.375 | –8.508E–04 | –1.093E–04 | –3.401E–04 | –3.407E–04 |
| 0.5 | –7.849E–04 | –1.022E–04 | –4.423E–04 | –4.431E–04 |
| 0.625 | –7.022E–04 | –9.294E–05 | –5.352E–04 | –5.362E–04 |
| 0.75 | –6.042E–04 | –8.165E–05 | –6.169E–04 | –6.180E–04 |
| 0.875 | –4.929E–04 | –6.857E–05 | –6.855E–04 | –6.867E–04 |
| 1 | –3.704E–04 | –5.395E–05 | –7.394E–04 | –7.407E–04 |
| 1.125 | –2.392E–04 | –3.810E–05 | –7.775E–04 | –7.789E–04 |
| 1.25 | –1.022E–04 | –2.134E–05 | –7.988E–04 | –8.003E–04 |
| 1.375 | 3.729E–05 | –4.059E–06 | –8.029E–04 | –8.044E–04 |
| 1.5 | 1.758E–04 | 1.332E–05 | –7.896E–04 | –7.910E–04 |
| 1.625 | 3.094E–04 | 3.032E–05 | –7.593E–04 | –7.606E–04 |
| 1.75 | 4.338E–04 | 4.645E–05 | –7.128E–04 | –7.141E–04 |
| 1.875 | 5.445E–04 | 6.114E–05 | –6.517E–04 | –6.527E–04 |
| 2 | 6.366E–04 | 7.382E–05 | –5.779E–04 | –5.787E–04 |
| 2.125 | 7.047E–04 | 8.383E–05 | –4.940E–04 | –4.946E–04 |
| 2.25 | 7.434E–04 | 9.051E–05 | –4.035E–04 | –4.037E–04 |
| 2.375 | 7.466E–04 | 9.313E–05 | –3.104E–04 | –3.102E–04 |
| 2.5 | 7.082E–04 | 9.093E–05 | –2.195E–04 | –2.188E–04 |
| 2.625 | 6.214E–04 | 8.310E–05 | –1.364E–04 | –1.352E–04 |
| 2.75 | 4.794E–04 | 6.880E–05 | –6.756E–05 | –6.577E–05 |
| 2.875 | 2.748E–04 | 4.713E–05 | –2.043E–05 | –1.795E–05 |
| 3 | 0.000E+00 | 1.717E–05 | –3.254E–06 | 0.000E+00 |

The numerical and exact results can be plotted below where the results are in such good agreement that the graphs are indistinguishable.

Solution continued on next page...

(b) The moment can be computed as

For comparison purposes, an exact, closed form solution can be developed by differentiating Eq. P21.36 to give

The following table uses divided differences of order O(h2) to evaluate the derivative. The fourth and fifth columns hold the numerical moment and the exact moment, respectively. Note that centered differences are used for the intermediate intervals and forward and backward differences are used for the first and last intervals, respectively. The numerical and exact results can be plotted below where the results are in such good agreement that the graphs are indistinguishable.

| x | (x) | d(x)/dx | M(x) | Mtrue |
| --- | --- | --- | --- | --- |
| 0 | –9.375E–04 | 6.782E–07 | 40.7 | 0.0 |
| 0.125 | –9.277E–04 | 1.553E–04 | 9320.7 | 9347.9 |
| 0.25 | –8.987E–04 | 3.080E–04 | 18478.7 | 18533.0 |
| 0.375 | –8.508E–04 | 4.552E–04 | 27311.2 | 27392.6 |
| 0.5 | –7.849E–04 | 5.943E–04 | 35655.4 | 35763.9 |
| 0.625 | –7.022E–04 | 7.225E–04 | 43348.5 | 43484.2 |
| 0.75 | –6.042E–04 | 8.371E–04 | 50227.9 | 50390.6 |
| 0.875 | –4.929E–04 | 9.355E–04 | 56130.6 | 56320.5 |
| 1 | –3.704E–04 | 1.015E–03 | 60894.1 | 61111.1 |
| 1.125 | –2.392E–04 | 1.073E–03 | 64355.5 | 64599.6 |
| 1.25 | –1.022E–04 | 1.106E–03 | 66352.0 | 66623.3 |
| 1.375 | 3.729E–05 | 1.112E–03 | 66720.9 | 67019.3 |
| 1.5 | 1.758E–04 | 1.088E–03 | 65299.5 | 65625.0 |
| 1.625 | 3.094E–04 | 1.032E–03 | 61924.9 | 62277.6 |
| 1.75 | 4.338E–04 | 9.406E–04 | 56434.5 | 56814.2 |
| 1.875 | 5.445E–04 | 8.111E–04 | 48665.4 | 49072.3 |
| 2 | 6.366E–04 | 6.409E–04 | 38454.9 | 38888.9 |
| 2.125 | 7.047E–04 | 4.273E–04 | 25640.2 | 26101.3 |
| 2.25 | 7.434E–04 | 1.676E–04 | 10058.6 | 10546.9 |
| 2.375 | 7.466E–04 | –1.409E–04 | –8452.7 | –7937.3 |
| 2.5 | 7.082E–04 | –5.009E–04 | –30056.4 | –29513.9 |
| 2.625 | 6.214E–04 | –9.153E–04 | –54915.4 | –54345.7 |
| 2.75 | 4.794E–04 | –1.387E–03 | –83192.3 | –82595.5 |
| 2.875 | 2.748E–04 | –1.917E–03 | –115049.9 | –114426.0 |
| 3 | 0.000E+00 | –2.479E–03 | –148738.6 | –150000.0 |

Solution continued on next page...

The shear can be computed as the derivative of the moment. For centered, finite divided differences, this can be represented as

(1)

or alternatively as the product of EI times the second derivative of the slope. For centered , finite divided differences, this can be represented as,

(2)

Note that for the last interval, the backward O(h2) formula from Fig. 21.4 is used as in

For comparison purposes, an exact, closed form solution can be developed by differentiating the moment solution to give

Solution continued on next page...

The following table displays the results. The fourth and sixth columns hold the resulting moment for Eqs. (1) and (2), respectively. The exact moment is displayed in column seven. Note that centered differences are used for the intermediate intervals and forward and backward differences are used for the first and last intervals, respectively.

| x | (x) | M(x) | V(x) = dM/dx | d2(x)/dx2 | V(x) | Vtrue |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | –9.375E–04 | 40.7 | 74728.7 | 1.270E–03 | 76193.6 | 75000.0 |
| 0.125 | –9.277E–04 | 9320.7 | 73752.2 | 1.237E–03 | 74240.5 | 74349.0 |
| 0.25 | –8.987E–04 | 18478.7 | 71961.8 | 1.205E–03 | 72287.3 | 72395.8 |
| 0.375 | –8.508E–04 | 27311.2 | 68706.6 | 1.151E–03 | 69032.1 | 69140.6 |
| 0.5 | –7.849E–04 | 35655.4 | 64149.3 | 1.075E–03 | 64474.8 | 64583.3 |
| 0.625 | –7.022E–04 | 43348.5 | 58289.9 | 9.769E–04 | 58615.5 | 58724.0 |
| 0.75 | –6.042E–04 | 50227.9 | 51128.5 | 8.576E–04 | 51454.0 | 51562.5 |
| 0.875 | –4.929E–04 | 56130.6 | 42664.9 | 7.165E–04 | 42990.5 | 43099.0 |
| 1 | –3.704E–04 | 60894.1 | 32899.3 | 5.537E–04 | 33224.8 | 33333.3 |
| 1.125 | –2.392E–04 | 64355.5 | 21831.6 | 3.693E–04 | 22157.1 | 22265.6 |
| 1.25 | –1.022E–04 | 66352.0 | 9461.8 | 1.631E–04 | 9787.3 | 9895.8 |
| 1.375 | 3.729E–05 | 66720.9 | –4210.1 | –6.474E–05 | –3884.5 | –3776.0 |
| 1.5 | 1.758E–04 | 65299.5 | –19184.0 | –3.143E–04 | –18858.5 | –18750.0 |
| 1.625 | 3.094E–04 | 61924.9 | –35460.1 | –5.856E–04 | –35134.5 | –35026.0 |
| 1.75 | 4.338E–04 | 56434.5 | –53038.2 | –8.785E–04 | –52712.7 | –52604.2 |
| 1.875 | 5.445E–04 | 48665.4 | –71918.4 | –1.193E–03 | –71592.9 | –71484.4 |
| 2 | 6.366E–04 | 38454.9 | –92100.7 | –1.530E–03 | –91775.2 | –91666.7 |
| 2.125 | 7.047E–04 | 25640.2 | –113585.1 | –1.888E–03 | –113259.5 | –113151.0 |
| 2.25 | 7.434E–04 | 10058.6 | –136371.5 | –2.267E–03 | –136046.0 | –135937.5 |
| 2.375 | 7.466E–04 | –8452.7 | –160460.1 | –2.669E–03 | –160134.5 | –160026.0 |
| 2.5 | 7.082E–04 | –30056.4 | –185850.7 | –3.092E–03 | –185525.2 | –185416.7 |
| 2.625 | 6.214E–04 | –54915.4 | –212543.4 | –3.537E–03 | –212217.9 | –212109.4 |
| 2.75 | 4.794E–04 | –83192.3 | –240538.2 | –4.004E–03 | –240212.7 | –240104.2 |
| 2.875 | 2.748E–04 | –115049.9 | –262185.3 | –4.492E–03 | –269509.5 | –269401.0 |
| 3 | 0.000E+00 | –148738.6 | –276833.8 | –4.980E–03 | –298806.4 | –300000.0 |

The numerical and exact results can be plotted below where the results are in such good agreement for Eq. (2) that the graphs are indistinguishable. In contrast, Eq. (1) does not do as well for the last point.