Problem 22.25

Figure P22.25 shows the kinetic interactions governing the concentrations of a bacteria culture and their nutrition source (substrate) in a continuously stirred flow-through bioreactor.

The mass balances for the bacteria biomass, X (gC∕m3), and the substrate concentration, S (gC/m3), can be written as

where t = time (h), kg,max = maximum bacterial growth rate (/d), Ks = half-saturation constant (gC/m3), kd = death rate (/d), kr = respiration rate (h), Q = flow rate (m3/h), V = reactor volume (m3), Y = yield coefficient (gC-cell/gC-substrate), and Sin = inflow substrate concentration (mgC/m3). Simulate how the substrate, bacteria, and total organic carbon (X + S) change over time in this reactor for three residence times:

(a) τw = 20 h,    (b) τw = 10 h, and     (c) τw = 5 h.

Employ the following parameters for the simulation: X(0) = 100 gC/m3, S(0) = 0, kg,max = 0.2/hr, Ks = 150 gC/m3, kd = kr = 0.01/hr, Y = 0.5 gC-cell/gC-substrate, V = 0.01 m3, and Sin = 1000 gC/m3, and display your results graphically.