Solution 22.6

We can solve this problem with the M-file Eulode (Fig. 22.3). First, we develop a function to compute the derivatives

function dydt = projectile(t,y,radius)

% y(1) = elevation (m)

% y(2) = velocity (m/s)

global g

dydt = [y(2);-g*radius^2/(radius+y(1))^2];

The RK4sys function (Fig. 22.8) can then be used to generate results and display them graphically as invoked by the following script:

clear,clc,format compact

global g

h=0.0625;tspan=[0 200];y0=[0 1500];

g=9.81;radius=6.37e6;

[t y] = rk4sys(@projectile,tspan,y0,h,radius);

max_elevation_km=max(y(:,1))/1e3

plot(t,y(:,1))

title('elevation (m) versus time (s)'),grid

Results:

max_elevation_km =

116.7813