Solution 22.12

A MATLAB M-file for the fourth-order RK method can be developed as

function [t,y] = rk4(dydt,tspan,y0,h)

% [t,y] = rk4(dydt,tspan,y0,h):

%   uses the fourth-order Runge-Kutta method to integrate an ODE

% input:

%   dydt = name of the M-file that evaluates the ODE

%   tspan = [ti, tf] where ti and tf = initial and

%           final values of independent variable

%   y0 = initial value of dependent variable

%   h = step size

% output:

%   t = vector of independent variable

%   y = vector of solution for dependent variable

ti = tspan(1);

tf = tspan(2);

t = (ti:h:tf)';

n = length(t);

% if necessary, add an additional value of t

% so that range goes from t = ti to tf

if t(n)<tf

t(n+1) = tf;

n = n+1;

end

y = y0*ones(n,1); %preallocate y to improve efficiency

for i = 1:n-1

hh = t(i+1) - t(i);

k1 = feval(dydt,t(i),y(i));

ymid = y(i) + k1*hh/2;

k2 = feval(dydt,t(i)+hh/2,ymid);

ymid = y(i) + k2*hh/2;

k3 = feval(dydt,t(i)+hh/2,ymid);

yend = y(i) + k3*hh;

k4 = feval(dydt,t(i)+hh,yend);

phi = (k1+2*(k2+k3)+k4)/6;

y(i+1) = y(i) + phi*hh;

end

plot(t,y)

Here is the test of the solution of Prob. 22.2. First, an M-file holding the differential equation is written as

function dy = dydx(x, y)

dy = (1+2*x)*sqrt(y);

Solution continued on next page...

Then the M-file can be invoked as in

>> [x,y]=rk4(@dydx,[0 1],1,0.1);

>> disp([x,y])

0    1.0000

0.1000    1.1130

0.2000    1.2544

0.3000    1.4280

0.4000    1.6384

0.5000    1.8906

0.6000    2.1904

0.7000    2.5440

0.8000    2.9584

0.9000    3.4410

1.0000    4.0000

The following plot is generated