Solution 22.23

Here is a function to implement the midpoint method:

function [tp,yp] = midpoint(dydt,tspan,y0,h,varargin)

% [t,y] = midpoint(dydt,tspan,y0,h):

%   uses the midpoint method to integrate an ODE

% input:

%   dydt = name of the M-file that evaluates the ODE

%   tspan = [ti, tf]; initial and final times with output

%          generated at interval of h, or

%         = [t0 t1 ... tf]; specific times where solution output

%   y0 = initial values of dependent variables

%   h = step size

%   p1,p2,... = additional parameters used by dydt

% output:

%   tp = vector of independent variable

%   yp = vector of solution for dependent variables

if nargin<4,error('at least 4 input arguments required'), end

if any(diff(tspan)<=0),error('tspan not ascending order'), end

n = length(tspan);

ti = tspan(1);tf = tspan(n);

if n == 2

t = (ti:h:tf)'; n = length(t);

if t(n)<tf

t(n+1) = tf;

n = n+1;

end

else

t = tspan;

end

tt = ti; y(1,:) = y0;

np = 1; tp(np) = tt; yp(np,:) = y(1,:);

i=1;

while(1)

tend = t(np+1);

hh = t(np+1) - t(np);

if hh>h,hh = h;end

while(1)

if tt+hh>tend,hh = tend-tt;end

k1 = dydt(tt,y(i,:),varargin{:})';

ymid = y(i,:) + k1*hh/2;

k2 = dydt(tt+hh/2,ymid,varargin{:})';

y(i+1,:) = y(i,:) + k2*hh;

tt = tt+hh;

i=i+1;

if tt>=tend,break,end

end

np = np+1; tp(np) = tt; yp(np,:) = y(i,:);

if tt>=tf,break,end

end

Solution continued on next page...

The following function holds the Lorenz ODEs:

function yp=lorenz(t,y,sigma,b,r)

yp=[-sigma*y(1)+sigma*y(2);r*y(1)-y(2)-y(1)*y(3);-b*y(3)+y(1)*y(2)];

The following script solves the ODEs and generates the plots:

clear,clc,clf

tspan=[0 20];y0=[5 5 5];

sigma=10;b=8/3;r=28;

[t y] = midpoint(@lorenz,tspan,y0,0.03125,sigma,b,r);

subplot(2,3,[1 2 3])

plot(t,y(:,1))

title('(a) Lorenz model x versus t');

subplot(2,3,4);plot(y(:,1),y(:,2))

xlabel('x');ylabel('y')

axis square;title('(b) y versus x')

subplot(2,3,5);plot(y(:,1),y(:,3))

xlabel('x');ylabel('z')

axis square;title('(c) z versus x')

subplot(2,3,6);plot(y(:,2),y(:,3))

xlabel('y');ylabel('z')

axis square;title('(d) z versus y')

pause

subplot(1,1,1)

plot3(y(:,1),y(:,2),y(:,2))

xlabel('x');ylabel('y');zlabel('z');grid

Here are the results of running the script:

Solution continued on next page...