Problem 23.1

Repeat the same simulations as in Sec. 23.5 for Pliny’s fountain, but generate the solutions with ode23, ode23s, and ode113. Use subplot to develop a vertical three-pane plot of the time series.

23.5 Case Study Problem Statement:

Background: The Roman natural philosopher, Pliny the Elder, purportedly had an intermittent fountain in his garden. As in Fig. 23.12, water enters a cylindrical tank at a constant flow rate Qin and fills until the water reaches yhigh. At this point, water siphons out of the tank through a circular discharge pipe, producing a fountain at the pipe’s exit. The fountain runs until the water level decreases to ylow, whereupon the siphon fills with air and the fountain stops. The cycle then repeats as the tank fills until the water reaches yhigh, and the fountain flows again. When the siphon is running, the outflow Qout can be computed with the following formula based on Torricelli’s law:

Neglecting the volume of water in the pipe, compute and plot the level of the water in the tank as a function of time over 100 seconds. Assume an initial condition of an empty tank y(0) = 0, and employ the following parameters for your computation:

RT = 0.05 m 	r = 0.007 m 	ylow = 0.025 m

yhigh = 0.1 m 	C = 0.6 	g = 9.81 m/s2

Qin = 50 × 10−6 m3/s