Problem 23.23

As depicted in Fig. P23.23, a double pendulum consists of a pendulum attached to another pendulum. We indicate the upper and lower pendulums by subscripts 1 and 2, respectively, and we place the origin at the pivot point of the upper pendulum with y increasing upward. We further assume that the system oscillates in a vertical plane subject to gravity, that the pendulum rods are massless and rigid, and the pendulum masses are considered to be point masses. Under these assumptions, force balances can be used to derive the following equations of motion where the subscripts 1 and 2 designate the top and bottom pendulum, respectively, θ = angle (radians) with 0 = vertical downward and counter-clockwise positive, t = time (s), g = gravitational acceleration (= 9.81 m/s2), m = mass (kg), and L = length (m). Note that the x and y coordinates of the masses are functions of the angles as in

x1 = L1 sinθ1 	y1 = −L1 cos θ1

x2 = x1 + L2 sin θ2 	y2 = y1 − L2 cos θ2

(a) Use ode45 to solve for the angles and angular velocities of the masses as a function of time from t = 0 to 40 s. Employ subplot to create a stacked plot with a time series of the angles in the top panel and a state space plot of θ2 versus θ1 in the bottom panel.

(b) Create an animated plot depicting the motion of the pendulum. Test your code for the following:

Case 1 (small displacement): L1 = L2 = 1 m, m1 = m2 = 0.25 kg, with initial conditions: θ1 = 0.5 m and θ2 = dθ1∕dt = dθ2∕dt = 0.

Case 2 (large displacement): L1 = L2 = 1 m, m1 = 0.5 kg, m2 = 0.25 kg, with initial conditions: 
θ1 = 1 m and θ2 = dθ1∕dt = dθ2∕dt = 0.

Problem continued on next page...