Problem 23.25

Develop a MATLAB script using ode45 to compute the velocity, υ, and position, z, of a hot air balloon as de-scribed in Prob. 23.24. Perform the calculation from t = 0 to 60 s with a step size of 1.6 s. At z = 200 m, assume that part of the payload (100 kg) is dropped out of the balloon. Develop a plot of your results.

Problem 23.24

Figure P23.24 shows the forces exerted on a hot air balloon system. Formulate the drag force as

where ρa = air density (kg/m3), υ = velocity (m/s), A = projected frontal area (m2), and Cd = the dimensionless drag co-efficient (≅ 0.47 for a sphere). Note also that the total mass of the balloon consists of two components:

m = mG + mP

where mG = the mass of the gas inside the expanded balloon (kg), and mP = the mass of the payload (basket, passengers, and the unexpanded balloon = 265 kg). Assume that the ideal gas law holds (P = ρRT), that the balloon is a perfect sphere with a diameter of 17.3 m, and that the heated air inside the envelope is at roughly the same pressure as the outside air. Other necessary parameters are normal atmospheric pressure, P = 101,300 Pa; gas constant for dry air, R = 287 Joules/kg · K; average temperature of air inside the balloon, T = 100 °C; and the normal (ambient) air density, ρ = 1.2 kg/m3.

(a) Use a force balance to develop the differential equation for dv∕dt as a function of the model’s fundamental parameters.

(b) At steady-state, calculate the particle’s terminal velocity.

(c) Use ode45 to compute the velocity and position of the balloon from t = 0 to 60 s given the previous parameters along with the initial condition: υ(0) = 0. Develop a plot of your results.

Problem continued on next page...