Problem 23.27

The growth of bacteria from substrate can be represented by the following pair of differential equations

where X = bacterial biomass, t = time (d), Y = a yield coefficient, kmax = maximum bacterial growth rate, S = substrate concentration, and ks = half saturation constant. The parameter values are Y = 0.75, kmax = 0.3, and ks = 1 × 10–4 and the initial conditions at t = 0 are S(0) = 5 and X(0) = 0.05. Note that neither X nor S can fall below zero as negative values are impossible.

(a) Use ode23 to solve for X and S from t = 0 to 20.

(b) Repeat the solution, but set the relative tolerance to 1 × 10–6.

(c) Keep repeating the solution with the relative tolerance set to 1 × 10–6, but determine which of the MATLAB ode solvers (including the stiff solvers) obtains correct (i.e., positive) results.

Use the tic and toc functions to determine the execution time for each option.