Problem 23.28

The oscillations of a swinging pendulum can be simulated with the following nonlinear model:

where θ = the angle of displacement (radians), g = the gravitational constant (= 9.81 m/s2), and l = the pendulum length.

(a) Express this equation as a pair of first-order ODEs.

(b) Use ode45 to solve for θ and dθ∕dt as a function of time for the case where l = 0.65 m and the initial conditions are θ = π∕8 and dθ∕dt = 0.

(c) Generate a plot of your results, and

(d) use the diff function to generate a plot of the angular accelerations (d2θ∕dt2) versus time based on the vector of angular velocities (dθ∕dt) generated in (b).

Use subplot to display all graphs as a single vertical three-panel plot with the top, middle, and bottom plots corresponding to θ, dθ∕dt, and d2θ∕dt2, respectively.