Solution 23.8

(a) The exact solution is

If the initial condition at t = 0 is 0.08, A = 0,

Note that even though the choice of the initial condition removes the positive exponential terms, it still lurks in the background. Very tiny round off errors in the numerical solutions bring it to the fore. Hence all of the following solutions eventually diverge from the analytical solution.

(b) 4th order RK. Here are the first few steps:

| t | y |
| --- | --- |
| 0 | 0.08 |
| 0.03125 | 0.093476 |
| 0.0625 | 0.108906 |
| 0.09375 | 0.126289 |
| 0.125 | 0.145625 |
| 0.15625 | 0.166914 |
| 0.1875 | 0.190156 |
| 0.21875 | 0.215351 |
| 0.25 | 0.242499 |

The plot shows the numerical solution (bold line) along with the exact solution (fine line).

Solution continued on next page...

(c)

function yp = dy(t,y)

yp = 5*(y-t^2);

>> tspan = [0,5];

>> y0 = 0.08;

>> [t,y] = ode45(@dy,tspan,y0);

(d)

>> [t,y] = ode23s(@dy,tspan,y0);

(e)

>> [t,y] = ode23tb(@dy,tspan,y0);