Solution 23.10

The nonlinear model can be expressed as the following set of ODEs,

where v = the angular velocity. A function can be developed to compute the right-hand-side of this pair of ODEs for the case where g = 9.81 and l = 0.6 m,

function dy = dpnon(t, y)

dy = [y(2);-9.81/0.6*sin(y(1))];

The linear model can be expressed as the following set of ODEs,

A function can be developed as,

function dy = dplin(t, y)

dy = [y(2);-9.81/0.6*y(1)];

Then, the solution and plot can be obtained for the case where (0) = /8. Note that we only depict the displacement ( or y(1)) in the plot

>> [tn yn] = ode45(@dpnon,[0 10],[pi/8 0]);

>> [tl yl] = ode45(@dplin,[0 10],[pi/8 0]);

>> plot(tn,yn(:,1),tl,yl(:,1),'--')

>> legend('nonlinear','linear')

Solution continued on next page...

You should notice two aspects of this plot. First, because the displacement is small, the linear solution provides a decent approximation of the more physically realistic nonlinear case. Second, the two solutions diverge as the computation progresses.

For the larger initial displacement ((0) = /2), the solution and plot can be obtained as,

>> [tn yn] = ode45(@dpnon,[0 10],[pi/2 0]);

>> [tl yl] = ode45(@dplin,[0 10],[pi/2 0]);

>> plot(tn,yn(:,1),tl,yl(:,1),'--')

>> legend('nonlinear','linear')

Because the linear approximation is only valid at small displacements, there are now clear and significant discrepancies between the nonlinear and linear cases that are exacerbated as the solution progresses.