Solution 23.13

In MATLAB, the first step is to set up a function to hold the differential equations:

function dc = dcdtstiff(t, c)

dc = [-0.013*c(1)-1000*c(1)*c(3);-2500*c(2)*c(3);-0.013*c(1)-1000*c(1)*c(3)-2500*c(2)*c(3)];

Then, an ODE solver like the function ode45 can be implemented as in

>> tspan=[0,50];

>> y0=[1,1,0];

>> [t,y]=ode45(@dcdtstiff,tspan,y0);

If this is done, the solution will take a relatively long time to compute the results. In contrast, because it is expressly designed to handle stiff systems, a function like ode23s yields results almost instantaneously.

>> [t,y]=ode23s(@dcdtstiff,tspan,y0);

In either case, a plot of the results can be developed as

>> plot(t,y)

Both plots will be identical as shown here