Solution 23.18

We can develop an M-file to compute the ODEs,

function yp = predpreylog(t,y,a,b,c,d,K)

yp = [a*(1-y(1)/K)*y(1)-b*y(1)*y(2);-c*y(2)+d*y(1)*y(2)];

The following script employs the ode45 function to generate the solution.

a=1.2;b=0.6;c=0.8;d=0.3;

[t y] = ode45(@predpreylog,[0 40],[2 1],[],a,b,c,d,1e8);

subplot(2,2,1);plot(t,y(:,1),t,y(:,2),'--')

title('(a) time series plot (K = 1e8)')

subplot(2,2,3);plot(y(:,1),y(:,2))

title('(b) phase plane plot (K = 1e8)')

[t y] = ode45(@predpreylog, [0 40],[2 1],[],a,b,c,d,200);

subplot(2,2,2);plot(t,y(:,1),t,y(:,2),'--')

title('(c) time series plot (K = 200)')

subplot(2,2,4);plot(y(:,1),y(:,2))

title('(d) phase plane plot (K = 200)')

Two things are indicated by these plots:

1. The period of the oscillations seems to be unaffected by the introduction of a carrying capacity effect.

2. The amplitudes of the oscillations decrease with time when a meaningful carrying capacity is imposed.

Solution continued on next page...

The second result might suggest a further question of whether or not the oscillations would eventually stabilize. This can be investigated by extending the integration interval as in the following script:

a=1.2;b=0.6;c=0.8;d=0.3;

[t y] = ode45(@predpreylog,[0 400],[2 1],[],a,b,c,d,200);

subplot(2,1,1);plot(t,y(:,1),t,y(:,2),'--')

title('(a) time series plot (K = 200)')

subplot(2,1,2);plot(y(:,1),y(:,2))

title('(b) phase plane plot (K = 200)')

The resulting plot indicates that the solution does not have stable oscillations but seems to be converging on stable constant populations.