Solution 23.19

The second-order equations can be expressed as the following system of first-order ODEs,

We can develop an M-file to compute them,

function dx = dxdtProb2319(t,x,k1,k2,m1,m2,w1,w2,L1,L2)

dx = [x(3);x(4);-k1/m1*(x(1)-L1)+k2/m1*(x(2)-x(1)-w1-L2);-k2/m2*(x(2)-x(1)-w1-L2)];

Solution continued on next page...

The following script employs the ode45 function to generate the solution.

k1=5;k2=5;m1=2;m2=2;w1=5;w2=5;L1=2;L2=2;

[t,x]=ode45(@dxdtProb2319,[0 20],[2 15 0 0],[],k1,k2,m1,m2,w1,w2,L1,L2);

subplot(2,1,1);plot(t,x(:,1),t,x(:,2),'--')

grid;title('displacements');legend('x1','x2')

subplot(2,1,2);plot(t,x(:,3),t,x(:,4),'--')

grid;title('velocities');legend('v1','v2')

pause

subplot(1,1,1),plot(x(:,2),x(:,1))

xlabel('x2');ylabel('x1')