Solution 23.21

The following function can be used to hold the ODEs:

function dx = dxdtProb2321(t,x,m1,m2,m3,k1,k2,k3)

dx=[x(4);x(5);x(6);-k1/m1*x(1)+k2/m1*(x(2)-x(1)); ...

k2/m2*(x(1)-x(2))+k3/m2*(x(3)-x(2)); ...

k3/m3*(x(2)-x(3))];

The following script employs the ode45 function to generate the solution and then generates the power spectrum for the floor’s displacement:

clear,clf,clc

m1=12000;m2=10000;m3=8000;

k1=3000;k2=2400;k3=1800;

tspan=[0:1/8:128];y0=[0 0 0 1 0 0];

[t,x]=ode45(@dxdtProb2321,tspan,y0,[],m1,m2,m3,k1,k2,k3);

subplot(2,1,1);plot(t,x(:,1))

xlabel('time (s)');ylabel('displacement (m)');

grid;title('(a) Displacement of first floor')

n=length(t)-1; dt=(max(t)-min(t))/n;fs=1/dt;nyquist=fs/2;

Y=fft(x(:,1))/n;

Y(1)=[];

% compute and display the power spectrum

f = (1:n/2)/(n/2)*nyquist;

Pyy = abs(Y(1:n/2)).^2;

subplot(2,1,2);

bar(f(1:n/32),Pyy(1:n/32))

title('(b) Power spectrum')

xlabel('Frequency (Hz)');

% Use eigenvalues to determine fundamental frequencies

Solution continued on next page...

The power spectrum indicates peaks at 0.0390625, 0.09375, and 0.1328125 Hz. This result can be validated by determining the systems eigenvalues:

A=[(k1+k2)/m1 -k2/m1 0; ...

-k2/m2 (k2+k3)/m2 -k3/m2; ...

0 -k3/m3 k3/m3];

lambda=eig(A);

freq=sqrt(lambda)/(2*pi)

freq =

0.1330

0.0927

0.0380