Solution 23.24

(a)

(b) Using Eq. (1) at steady state

(c)

% Hot Air Balloon Script

clear,clc,clf

format compact

global g

% set parameters

g=9.81;

r=8.65; % balloon radius

CD=0.47; % dimensionless drag coefficient

mP=265; % mass of payload

P=101300; % atmospheric pressure

Rgas=287; % Universal gas constant for dry air

TC=100; % air temperature

rhoa=1.2; % air density

ti=0; % initial time (s)

tf=60; % final time (s)

vi=0; % initial velocity

zi=0; % initial elevation

Solution continued on next page...

% precomputations

d = 2 * r; Ta = TC + 273.15; Ab = pi / 4 * d ^ 2;

Vb = pi / 6 * d ^ 3; rhog = P / Rgas / Ta; mG = Vb * rhog;

FB = Vb * rhoa * g; FG = mG * g; cdp = rhoa * Ab * CD / 2;

% compute times, velocities and elevations

tspan=[ti tf]; yinit=[vi zi];

[t,y]=ode45(@dydt2324c,tspan,yinit,[],FB,FG,mG,cdp,mP);

% Display results

fprintf('      t(s)     v(m/s)     z(m/s)\n');

xx=[t y];

fprintf('%10.3f %10.3f %10.3f\n',xx');

% Plot results

subplot(2,1,1)

plot(t,y(:,1))

title('Velocity vs. time')

xlabel('time (s)'),ylabel('velocity (m/s)')

subplot(2,1,2)

plot(t,y(:,2))

title('Elevation vs. time')

xlabel('time (s)'),ylabel('elevation (m)')

function dy=dydt2324c(t,y,FB,FG,mG,cdp,mP)

global g

dy = [(FB - FG) / (mP + mG) - mP * g / (mP + mG) - ...

cdp / (mP + mG) * y(1) ^ 2; y(1)];

t(s)     v(m/s)     z(m/s)

0.000      0.000      0.000

0.000      0.000      0.000

0.000      0.000      0.000

0.000      0.000      0.000

0.000      0.000      0.000

0.000      0.000      0.000

0.000      0.001      0.000

0.001      0.001      0.000

0.001      0.001      0.000

0.002      0.002      0.000

0.003      0.004      0.000

0.003      0.005      0.000

0.004      0.006      0.000

. .        . .        . .

. .        . .        . .

. .        . .        . .

53.943      7.920    397.677

55.443      7.920    409.557

56.943      7.920    421.438

57.707      7.920    427.491

58.472      7.920    433.544

59.236      7.920    439.597

60.000      7.920    445.650

Solution continued on next page...

Notice that the terminal velocity (7.92) confirms the result computed analytically in (b). Such checks are useful in verifying that you numerical results are valid.