Solution 23.27

clear,clc,clf

Y=0.75;kmax=0.3;ks=1e-4;

tspan=[0 25];y0=[0.05 5];

disp('ode23');

tic;[t, y] = ode23(@bacteria,tspan,y0,[],Y,kmax,ks);toc;

plot(t,y(:,1),t,y(:,2),'--'),legend('X (bacteria)','S (substrate)')

title('(a) time series plot')

pause

disp('ode23 with tol = 1e-6');

options=odeset('RelTol',1e-6);

tic;[t, y] = ode23(@bacteria,tspan,y0,options,Y,kmax,ks);toc;

plot(t,y(:,1),t,y(:,2),'--'),legend('X (bacteria)','S (substrate)')

title('(a) time series plot')

pause

disp('ode45 with tol = 1e-6');

tic;[t, y] = ode45(@bacteria,tspan,y0,options,Y,kmax,ks);toc;

plot(t,y(:,1),t,y(:,2),'--'),legend('X (bacteria)','S (substrate)')

title('(a) time series plot')

pause

disp('ode15s with tol = 1e-6');

tic;[t, y] = ode15s(@bacteria,tspan,y0,options,Y,kmax,ks);toc;

plot(t,y(:,1),t,y(:,2),'--'),legend('X (bacteria)','S (substrate)')

title('(a) time series plot')

pause

disp('ode23s with tol = 1e-6');

tic;[t, y] = ode23s(@bacteria,tspan,y0,options,Y,kmax,ks);toc;

plot(t,y(:,1),t,y(:,2),'--'),legend('X (bacteria)','S (substrate)')

title('(a) time series plot')

pause

disp('ode23t with tol = 1e-6');

tic;[t, y] = ode23t(@bacteria,tspan,y0,options,Y,kmax,ks);;toc;

plot(t,y(:,1),t,y(:,2),'--'),legend('X (bacteria)','S (substrate)')

title('(a) time series plot')

pause

disp('ode23tb with tol = 1e-6');

tic;[t, y] = ode23tb(@bacteria,tspan,y0,options,Y,kmax,ks);toc;

plot(t,y(:,1),t,y(:,2),'--'),legend('X (bacteria)','S (substrate)')

title('(a) time series plot')

function yp = bacteria(t,y,Y,kmax,ks)

yp = [Y*kmax*y(2)/(ks+y(2))*y(1);-kmax*y(2)/(ks+y(2))*y(1)];

end

Solution continued on next page...

Results:

ode23

Elapsed time is 0.024489 seconds.

ode23 with tol = 1e-6

Elapsed time is 1.835925 seconds.

ode45 with tol = 1e-6

Elapsed time is 3.070684 seconds.

ode15s with tol = 1e-6

Elapsed time is 0.077614 seconds.

ode23s with tol = 1e-6

Elapsed time is 0.105561 seconds.

ode23t with tol = 1e-6

Elapsed time is 0.072509 seconds.

ode23tb with tol = 1e-6

Elapsed time is 0.069694 seconds.

The first option yields incorrect negative results, but does so quickly.

The remaining functions all yield positive results, but the non-stiff solvers (ode23 and ode45) take significantly longer to execute. Notice that the ode45 takes the longest. All the stiff solvers succeed quickly.