Solution 23.31

This is an initial-value problem because the values of the variables are given at the start of the integration interval; i.e., at x = 0, y = z = 0. In order to obtain a numerical solution, the second-order differential equation can be expressed as a pair of first-order ODEs,

These equations can be simply integrated with any of the techniques from this part of the book.

The following shows the results of using ode45 to obtain the solution.

clear,clc

E = 2e11; I = 0.00033; P = 4.5e3;L = 3;

xspan = [0:L/100:L]; y0 = [0 0];

[x,y]=ode45(@cantilever,xspan,y0,[],E,I,P,L);

subplot(2,1,1)

plot(x,y(:,1))

ylabel('m')

subplot(2,1,2)

plot(x,y(:,2))

xlabel('distance (m)')

ylabel('m/m')

function dy=cantilever(x,y,E,I,P,L)

dy=[y(2);-P/(E*I)*(L-x)];