Problem 24.12

The following differential equation describes the steady-state concentration of a substance that reacts with first-order kinetics in an axially dispersed plug-flow reactor (Fig. P24.12):

where D = the dispersion coefficient (m2/hr), c = concentration (mol/L), x = distance (m), U = the velocity (m/hr), and k = the reaction rate (/hr). The boundary conditions can be formulated as

where cin = the concentration in the inflow (mol/L), L = the length of the reactor (m). These are called Danckwerts boundary conditions.

Use the finite-difference approach to solve for concentration as a function of distance given the following parameters: D = 5000 m2/hr, U = 100 m/hr, k = 2/hr, L = 100 m, and cin = 100 mol/L. Employ centered finite-difference approximations with Δ x = 10 m to obtain your solutions. Compare your numerical results with the analytical solution:

where