Problem 24.13

A series of first-order, liquid-phase reactions create a desirable product (B) and an undesirable byproduct (C):

If the reactions take place in an axially dispersed plug-flow reactor (Fig. P24.12), steady-state mass balances can be used to develop the following second-order ODEs:

Use the finite-difference approach to solve for the concentration of each reactant as a function of distance given: D = 0.1 m2/min, U = 1 m/min, k1 = 3/min, k2 = 1/min, L = 0.5 m, ca,in = 10 mol/L. Employ centered finite-difference approximations with Δ x = 0.05 m to obtain your solutions and assume Danckwerts boundary conditions as described in Prob. 24.12. Also, compute the sum of the reactants as a function of distance. Do your results make sense?