Problem 24.14

A biofilm with a thickness Lf (cm), grows on the surface of a solid (Fig. P24.14). After traversing a diffusion layer of thickness L (cm), a chemical compound A diffuses into the biofilm where it is subject to an irreversible first-order reaction that converts it to a product B.

Steady-state mass balances can be used to derive the following ordinary differential equations for compound A:

where D = the diffusion coefficient in the diffusion layer = 0.8 cm2/d, Df = the diffusion coefficient in the biofilm = 0.64 cm2/d, and k = the first-order rate for the conversion of A to B = 0.1/d. The following boundary conditions hold:

where ca0 = the concentration of A in the bulk liquid = 100 mol/L. Use the finite-difference method to compute the steady-state distribution of A from x = 0 to L + L f, where L = 0.008 cm and 
Lf = 0.004 cm. Employ centered finite differences with Δ x = 0.001 cm.