Problem 24.16

The basic differential equation of the elastic curve for a simply supported, uniformly loaded beam (Fig. P24.16) is given as

where E = the modulus of elasticity and I = the moment of inertia. The boundary conditions are y(0) = y(L) = 0. Solve for the deflection of the beam using

(a) the finite-difference approach (Δ x = 0.6 m) and

(b) the shooting method.

The following parameter values apply: E = 200 GPa, I = 30,000 cm4, 𝑤 = 15 kN/m, and L = 3 m. Compare your numerical results to the analytical solution: