Problem 24.17

In Prob. 24.16, the basic differential equation of the elastic curve for a uniformly loaded beam was formulated as

Note that the right-hand side represents the moment as a function of x. An equivalent approach can be formulated in terms of the fourth derivative of deflection as

For this formulation, four boundary conditions are required. For the supports shown in Fig. P24.16, the conditions are that the end displacements are zero, y(0) = y(L) = 0, and that the end moments are zero, y″(0) = y″(L) = 0. Solve for the deflection of the beam using the finite-difference approach (Δx = 0.6 m). The following parameter values apply: E = 200 GPa, I = 30,000 cm4, 𝑤 = 15 kN/m, and L = 3 m. Compare your numerical results with the analytical solution given in Prob. 24.16.

Problem continued on next page...

Problem 24.16

The basic differential equation of the elastic curve for a simply supported, uniformly loaded beam (Fig. P24.16) is given as

where E = the modulus of elasticity and I = the moment of inertia. The boundary conditions are y(0) = y(L) = 0. Solve for the deflection of the beam using

(a) the finite-difference approach (Δ x = 0.6 m) and

(b) the shooting method.

The following parameter values apply: E = 200 GPa, I = 30,000 cm4, 𝑤 = 15 kN/m, and L = 3 m. Compare your numerical results to the analytical solution: