Solution 24.15

Main Program:

% Hanging static cable - w=w(x)

% Parabolic solution w=w(x)

% CUS Units (kg,m,s)

% w = wo(1+sin(pi/2*x/l)

% Independent Variable x

% initial conditions [y(1)=cable y-coordinate, y(2)=cable slope];

clear,clc,format compact

global wToP

es=0.5e-7; w0=450;

xspan=[0,60]; ic=[0 0];

wToP=0.0025;

[x,y]=ode45(@slp,xspan,ic);

yf(1)=y(length(x));

wTo(1)=wToP;

ea(1)=1;

wToP=0.002;

[x,y]=ode45(@slp,xspan,ic);

yf(2)=y(length(x));

wTo(2)=wToP;

ea(2)=abs((yf(2)-yf(1))/yf(2));

for k=3:10

wTo(k)=wTo(k-1)+(wTo(k-1)-wTo(k-2))/(yf(k-1)-yf(k-2))*(15-yf(k-1));

wToP=wTo(k);

[x,y]=ode45(@slp,xspan,ic);

yf(k)=y(length(x));

ea(k)=abs( (yf(k)-yf(k-1))/yf(k) );

if (ea(k)<=es)

plot(x,y(:,1)); grid;

xlabel('x-coordinate - m'); ylabel('y-coordinate - m');

title('Cable - w=wo(1+sin(pi/2*x/l))');

break

end

end

wToP

To=w0/wToP

Function ‘slp’:

function dxy=slp(x,y)

global wToP

dxy=[y(2);(wToP)*(1+sin(pi/2*x/60))];

Solution continued on next page...

Results:

wToP =

0.0057

To =

7.8984e+04